# 1. Overview

## 1.1 Purpose

The Verification module in FlowBot Hub is designed to:

- Import ICM-exported trace files for DWF and Storm events.

- Allow modellers to compare observed vs predicted depth/flow (and optionally velocity).

- Automatically calculate verification metrics (peak differences, volume, shape / NSEC, etc.) against configurable tolerances based on the A7P Hydraulic Model Verification SOP.

- Enable interactive tweaking of peak selection and smoothing (within the tool) to refine scores.

- Support an iterative workflow where multiple traces (from different ICM runs) can be imported and compared for the same event, until a final verification run is agreed.

## 1.2 Scope

In scope:

- DWF and Storm verification only.

- Import of:

  - Observed & predicted time series (depth, flow, velocity if available).

  - DWF high/low envelope series (produced by Flow Survey Analysis).

- Tolerance-based scoring for:

  - Peak depth

  - Peak flow

  - Volume

  - Shape (NSEC / similar)

- Differentiation of:

  - General vs critical locations

  - Surcharged vs unsurcharged cases

- Storage of:

  - Multiple verification attempts per event per monitor.

  - Manual adjustments & overrides.

  - Marking “final” runs.

Out of scope (for this module):

- Historical verification (flooding / spills vs historical records).

- Derivation of DWF envelopes (handled by Flow Survey Analysis).

- Flow survey management & analysis (FSM/FSA) data reuse – no direct entity-level linking.

# 2. Roles & Permissions

### Entities already in global model

- User

- VerificationProject

- VerificationProjectCollaborator

# 3. Core Concepts & Data Model

## 3.1 High-level structure

- A VerificationProject groups all verification work.

- A project has one or more VerificationEvents (Storm A, Storm B, DWF weekday/weekend, full period runs, etc.).

- For each event, the modeller can import multiple VerificationTraceSets (ICM exports from different runs).

- Each TraceSet contains multiple MonitorTraceVersions (one per monitor/page in the export).

- Each monitor in the project is a VerificationFlowMonitor (project-local; not reused from FSM/FSA).

- For each MonitorTraceVersion, modellers create VerificationRuns (draft/final) with associated VerificationMetricSet & ManualAdjustment entries.

- Various TimeSeries entries back the actual observed/predicted/envelope data.

- ToleranceSet defines the rules for scoring by event type, criticality, and surcharging.

## 3.2 Entities (with key fields)

### 3.2.1 VerificationProject

Represents a single verification job (e.g. “Catchment XYZ – 2025 ICS Verification”).

Relationships:

- VerificationProject → VerificationProjectCollaborator (1–many)

- VerificationProject → VerificationFlowMonitor (1–many)

- VerificationProject → VerificationEvent (1–many)

- VerificationProject → VerificationTraceSet (1–many)

- VerificationProject → ToleranceSet (1–many)

### 3.2.2 VerificationProjectCollaborator

### 3.2.3 VerificationFlowMonitor

Project-scoped representation of a flow monitor for verification purposes.

- id

- verification_project_id → VerificationProject

- name (user-friendly label; e.g. M01 – High Street)

- icm_reference (string; node/monitor ID from ICM trace header if available)

- location_status (string; general, critical, surcharged).

- Optional:

  - location_x, location_y (if you want map linking later)

  - note

Used only inside Verification module (no shared FlowMonitor entity across FSM/FSA).

### 3.2.4 VerificationEvent

Conceptual event (e.g. Storm A, DWF weekday).

- id

- verification_project_id → VerificationProject

- name (e.g. Storm A, Storm B, DWF – weekday, Full period run)

- event_type (STORM, DWF)

- description (optional)

- is_active (bool)

- created_at

Relationships:

- VerificationEvent → VerificationTraceSet (1–many)

- VerificationEvent → (through TraceSets) → MonitorTraceVersion

### 3.2.5 VerificationTraceSet

One import of an ICM export file, representing a single event run.

- id

- verification_project_id → VerificationProject

- verification_event_id → VerificationEvent

- file_name

- file_hash (for deduping / reimport detection)

- imported_at

- imported_by_user_id → User

- status (ACTIVE, DELETED, ERROR)

- raw_file_storage_key (if the original file is stored somewhere)

- parser_version (for backwards compatibility)

Relationships:

- VerificationTraceSet → MonitorTraceVersion (1–many)

### 3.2.6 MonitorTraceVersion

Represents observed vs predicted series for a single monitor in a single trace import.

- id

- verification_trace_set_id → VerificationTraceSet

- verification_flow_monitor_id → VerificationFlowMonitor

- monitor_name_in_trace (string as found in the file; useful for matching)

- event_type (STORM, DWF) – should match parent event

- simulation_label (string; e.g. ICM run name)

- Time series references:

  - obs_depth_series_id → TimeSeries

  - obs_flow_series_id → TimeSeries

  - obs_velocity_series_id → TimeSeries

  - pred_depth_series_id → TimeSeries

  - pred_flow_series_id → TimeSeries

  - pred_velocity_series_id → TimeSeries

  - For DWF only:

    - dwf_high_envelope_series_id → TimeSeries

    - dwf_low_envelope_series_id → TimeSeries

- notes (text)

- created_at

Relationships:

- MonitorTraceVersion → VerificationRun (1–many)

### 3.2.7 TimeSeries

Generic time series store for verification.

Implementation detail is flexible; could be row-per-point or array/Parquet storage.

- id

- verification_project_id

- series_type (OBS_DEPTH, OBS_FLOW, OBS_VEL, PRED_DEPTH, PRED_FLOW, PRED_VEL, DWF_HIGH, DWF_LOW)

- unit (e.g. m, L/s)

- time_zone (optional)

- start_time / end_time

- data_store_type (ROW, PARQUET, BLOB) – how/where data is stored

- data_location (path/key)

- For minimal version: a JSON array of {timestamp, value}.

### 3.2.8 ToleranceSet

Configurable tolerances based on SOP.

- id

- verification_project_id

- name (e.g. Storm – General, Storm – Critical, DWF – General)

- event_type (STORM, DWF)

- applies_to_critical (bool)

- surcharged_mode (SURCHARGED, UNSURCHARGED, ANY)

- Depth tolerance:

  - peak_depth_min_diff_m

  - peak_depth_max_diff_m

  - OR store both absolute and relative values if needed.

- Flow tolerance:

  - peak_flow_min_percent

  - peak_flow_max_percent

- Volume tolerance:

  - volume_min_percent

  - volume_max_percent

- Shape tolerance:

  - nsec_min_threshold (e.g. > 0.5)

- source (e.g. A7P v1, Custom project config)

The default sets will be derived from the SOP tolerances for general vs critical and surcharged vs unsurcharged.

### 3.2.9 VerificationRun

A single scoring session for one MonitorTraceVersion (i.e. one attempt to score a specific trace for a specific monitor).

- id

- monitor_trace_version_id → MonitorTraceVersion

- tolerance_set_id → ToleranceSet

- created_at

- created_by_user_id → User

- status (DRAFT, FINAL, SUPERSEDED)

- is_final_for_monitor_event (bool) – enforce via constraint per (monitor + event)

- override_used (bool; if any ManualAdjustment exists)

- Aggregated results:

  - overall_status (VERIFIED, MARGINAL, NOT_VERIFIED, NOT_ASSESSED)

  - overall_score_depth

  - overall_score_flow

  - overall_score_volume

  - overall_score_shape (optional)

- comments (text)

Relationships:

- VerificationRun → VerificationMetricSet (1–many)

- VerificationRun → ManualAdjustment (1–many)

### 3.2.10 VerificationMetricSet

Stores calculated metrics and scoring per parameter.

- id

- verification_run_id → VerificationRun

- parameter (DEPTH, FLOW, VOLUME, SHAPE)

- Raw metrics:

  - peak_obs_value

  - peak_obs_time

  - peak_pred_value

  - peak_pred_time

  - peak_diff_abs

  - peak_diff_percent

  - time_offset_hours

  - volume_obs

  - volume_pred

  - volume_diff_percent

  - nsec_value (for SHAPE)

  - fraction_within_dwf_envelope (for DWF only)

- Derived scoring:

  - score_value (0, 2, 3)

  - score_band (OK, FAIR, NO, NA) – matches SOP scoring framework.

  - used_in_total (bool; in case of excluding metrics)

- Optional:

  - auto_peak_selection_method (algorithm used)

  - data_quality_flag (GOOD, POOR, MISSING)

### 3.2.11 ManualAdjustment

Captures user’s interactive tweaks to the automatically computed metrics.

- id

- verification_run_id → VerificationRun (or directly to VerificationMetricSet)

- parameter (DEPTH, FLOW, VOLUME, SHAPE)

- adjustment_type (PEAK_POINT_SELECT, SMOOTHING, EXCLUDE_PERIOD, etc.)

- original_peak_time

- original_peak_value

- adjusted_peak_time

- adjusted_peak_value

- Optional:

  - excluded_time_ranges (JSON list of intervals)

- reason (text)

- created_at

- created_by_user_id

On save, the system recalculates the corresponding VerificationMetricSet and flags override_used.

# 4. Key Workflows

## 4.1 Create Verification Project

- User clicks “New Verification Project”.

- Enters:

  - Project name

  - Description

- System creates VerificationProject, sets user as owner.

- Optional: Add collaborators.

## 4.2 Define Events

From project view:

- User goes to Events tab.

- Clicks “Add Event”:

  - Name: Storm A

  - Type: STORM

  - Description: optional.

- Repeat for Storm B, DWF weekday, etc.

Events can also be created on-the-fly during first trace import.

## 4.3 Import Trace (Event & Monitors)

- User clicks “Import Trace”.

- Upload ICM export .csv  (existing implementation using method getTracesFromCSVFile in _legacy_code\flowbot_verification.py)

- System parses:

  - Identifies page breaks / sections.

  - Extracts header details (monitor identifiers, event names if present).

  - Reads column structure (assumed consistent across pages).

  - Constructs list of candidate monitors: monitor_name_in_trace.

- Event selection step:

  - User chooses:

    - Match to existing VerificationEvent, OR

    - Create new event (name + type).

- Monitor matching step:

  - For each discovered monitor:

    - Auto-suggest a match to existing VerificationFlowMonitor by name / icm_reference.

    - Or offer “Create new monitor”.

  - User confirms mapping.

- System creates:

  - A VerificationTraceSet (linked to selected event).

  - A MonitorTraceVersion for each matched monitor, with associated TimeSeries for obs/pred data.

  - If DWF and envelope data is included in the trace or a separate file:

    - Link dwf_high_envelope_series_id / dwf_low_envelope_series_id.

- User sees import summary.

## 4.4 Verification Workspace (per monitor + trace)

From Event → Monitor or from a “coverage matrix” view:

- User selects:

  - Event (e.g. Storm A)

  - Monitor (e.g. M01)

  - TraceSet (e.g. ICM Run v3)

- System:

  - Loads MonitorTraceVersion.

  - Creates a new VerificationRun (status DRAFT), selecting appropriate ToleranceSet based on:

    - event_type

    - monitor.is_critical

    - is_surcharged_for_this_event

- System:

  - Auto-detects peaks / volumes / NSEC / DWF envelope compliance as needed.

  - Creates VerificationMetricSet entries for each parameter.

  - Aggregates scores to set overall_status.

- UI shows:

  - Chart with observed vs predicted series.

  - Peak markers for each parameter.

  - Table of metrics (time offset, peak diff, volume diff, NSEC, etc.).

  - Score bands (OK/Fair/No).

## 4.5 Adjusting Peaks / Metrics

In the same workspace:

- User toggles “Adjust peaks” mode.

- For depth / flow:

  - User can click a different point on the time series to set a new peak.

- For DWF:

  - User may choose to exclude periods or adjust smoothing options.

- System:

  - Records a ManualAdjustment.

  - Recalculates the corresponding VerificationMetricSet.

  - Recalculates aggregated VerificationRun scores.

  - Flags override_used = true.

User can save as draft or continue tweaking.

## 4.6 Finalising a Run

When satisfied:

- User clicks “Mark as Final”.

- System validates:

  - No other VerificationRun for the same (monitor_trace_version_id) (or for same (monitor, event) depending on how strict you want) has status FINAL.

- Sets:

  - status = FINAL

  - is_final_for_monitor_event = true

- Optionally marks other runs as SUPERSEDED.

Final runs are locked from further edits (except maybe comments).

## 4.7 Iterating with New Traces

When the modeller refines the ICM model:

- They re-export traces from ICM.

- Import into the same event.

- Now the Event view shows multiple TraceSets for that monitor/event.

- They repeat the verification workflow; can compare runs visually:

  - tables

  - charts with multiple predicted series (optional overlay).

## 4.8 Reporting / Export

From project level and event level:

- Verification Matrix view:

  - Monitors × Events × Parameters grid.

  - Colours: green/yellow/red/white, based on VerificationMetricSet.score_band.

  - Roll-up to monitor-level “verified/marginal/not verified”.

- Exports:

  - CSV / Excel report summarising:

    - For each monitor & event:

      - Final run metrics & scores

      - Overall status

      - Notes/overrides

  - Optional:

    - JSON export for machine use.

  - PDF?

# 5. Algorithms & Logic

## 5.1 Parsing ICM Export

Assumptions:

- Column layout is consistent for all pages in a given file.

- Each page has a header with monitor/location identifier.

- Columns likely:

  - Date, Time, Observed Depth, Observed Flow, Observed Velocity, Predicted Depth, Predicted Flow, Predicted Velocity, etc.

  - Additional predicted lines if multiple traces were plotted.

Implementation ideas:

- Identify header lines and use them to define a schema for the whole file.

- Identify page breaks based on known separator pattern (e.g. blank lines or specific text).

- For each page:

  - Extract monitor_name_in_trace from a title or specific column.

  - Create a MonitorTraceVersion.

  - Populate TimeSeries for obs/pred.

If multiple predicted traces per page exist: either choose one for verification or support mapping them to different MonitorTraceVersion entries (v2 feature).

## 5.2 Peak Detection

Basic approach:

- Clarify which metric you want:

  - Global max over the event window (common).

  - Or within some window (e.g. exclude noisy tails).

Algorithm:

- For a given TimeSeries (e.g. predicted depth):

  - Apply optional smoothing (configurable).

  - Find the time of max value.

- For observed series:

  - Same method.

- Compute:

  - peak_diff_abs = pred_peak - obs_peak

  - peak_diff_percent

  - time_offset_hours = (pred_peak_time - obs_peak_time) in hours.

Expose smoothing window parameters as config in AnalysisConfig-like table later if needed.

## 5.3 DWF Envelope Compliance

Given:

- DWF_HIGH(t) and DWF_LOW(t) series.

- Observed / predicted DWF series.

Algorithm:

- For each timestamp where both envelope and series are defined:

  - Check if series(t) lies between [DWF_LOW(t), DWF_HIGH(t)].

- Compute:

  - fraction_within_dwf_envelope = count(in_window) / count(total)

Then define thresholds such as:

- >= 0.90 → OK

- >= 0.75 → FAIR

- else → NO

Store these thresholds either in ToleranceSet or a separate config.

## 5.4 Tolerance & scoring

From SOP:

- For each parameter (Depth, Flow, Volume):

  - Use the appropriate tolerance bounds depending on:

    - event_type (DWF/STORM)

    - is_critical (true/false)

    - is_surcharged_for_this_event.

Apply:

- If within spec → OK = 3 points.

- If within a “near miss” margin (e.g. 0.1m beyond depth threshold, 5% beyond Flow/Volume threshold) → FAIR = 2 points.

- Else → NO = 0 points.

- Missing/unreliable → NA = 0 points.

Aggregate per parameter (for events where you verify multiple storms – three per monitor):

- For each parameter, sum the 3 scores across the event set:

  - ≥ 8 → VERIFIED

  - 6–7 → MARGINAL

  - ≤ 5 → NOT_VERIFIED.

In your implementation, you might encode this as:

- VerificationGroup (monitor + parameter + set of events)

- But for now you can keep it simple: treat each VerificationEvent as “storm group” and adapt as needed.

# 6. API Sketch (FastAPI-flavoured)

Just the gist:

- POST /verification-projects/

- GET /verification-projects/{id}

- POST /verification-projects/{id}/events/

- GET /verification-projects/{id}/events/

- POST /verification-projects/{id}/trace-sets/import

  - multipart file + event_id or new_event parameters

- GET /verification-trace-sets/{id}

- GET /events/{event_id}/monitors

- GET /monitor-trace-versions/{id}

- POST /monitor-trace-versions/{id}/verification-runs/

- PATCH /verification-runs/{id} (e.g. mark final)

- POST /verification-runs/{id}/adjustments/

- GET /verification-runs/{id}

- GET /verification-projects/{id}/matrix (summary matrix)

# 7. Front-end Views (React)

- Project List

  - list of VerificationProjects.

- Project Overview

  - tabs:

    - Events

    - Monitors

    - Imports

    - Matrix

    - Settings (tolerances)

- Import Wizard

  - upload → event selection → monitor matching → summary.

- Event View

  - list of TraceSets

  - for each monitor: status badges (verified, marginal, unverified).

- Verification Workspace (monitor + trace)

  - time series chart (obs vs pred, maybe envelope)

  - metrics table

  - adjustments panel

  - “Save draft” / “Mark final”

- Matrix View

  - monitors vs events

  - colour-coded cells per parameter (Depth, Flow, Volume, Shape)

  - filter by parameter / status.